# PowerGuard — Complete Deep Dive Study Guide

Electricity Theft Detection System

# 🧠 PowerGuard — Complete Deep Dive Study Guide

This guide is written assuming you have **zero prior knowledge**. Read this carefully, and you will be able to confidently explain every single technical concept in your project — from the physics of electricity theft to the math behind every model.

---

## Table of Contents

1. [The Core Problem — NTL & Electricity Theft](#1-the-core-problem--ntl--electricity-theft)

2. [The Dataset — SGCC](#2-the-dataset--sgcc)

3. [The 6 Input Dimensions — What the Smart Meter Sends](#3-the-6-input-dimensions--what-the-smart-meter-sends)

4. [⭐ The 8 Heuristic Detection Factors — DEEP DIVE](#4-the-8-heuristic-detection-factors--deep-dive)

5. [How the 8 Factors Combine — Weighted Scoring & Boosting](#5-how-the-8-factors-combine--weighted-scoring--boosting)

6. [Feature Engineering — The 47+ Engineered Features](#6-feature-engineering--the-47-engineered-features)

7. [Class Imbalance — Why It's Hard & How We Solved It](#7-class-imbalance--why-its-hard--how-we-solved-it)

8. [The 5 Machine Learning Models](#8-the-5-machine-learning-models)

9. [The Ensemble Strategy — Soft Voting & Stacking](#9-the-ensemble-strategy--soft-voting--stacking)

10. [Explainability — SHAP & the Heuristic Breakdown](#10-explainability--shap--the-heuristic-breakdown)

11. [The Training Pipeline — End to End](#11-the-training-pipeline--end-to-end)

12. [6 Theft Scenarios We Simulate](#12-6-theft-scenarios-we-simulate)

13. [Evaluation Metrics — How We Prove It Works](#13-evaluation-metrics--how-we-prove-it-works)

14. [System Architecture — 4 Microservices](#14-system-architecture--4-microservices)

15. [The Triple-Layer Fallback — Why the System Never Fails](#15-the-triple-layer-fallback--why-the-system-never-fails)

16. [Data Flow — From Button Click to Result](#16-data-flow--from-button-click-to-result)

17. [Risk Classification — 4 Tiers](#17-risk-classification--4-tiers)

18. [Summary to Memorize](#18-summary-to-memorize)

---

## 1. The Core Problem — NTL & Electricity Theft

### What is NTL?

**NTL = Non-Technical Loss.** In power distribution, there are two types of losses:

Type | Cause | Example | Can ML Detect?

**Technical Loss** | Physics | Heat in wires, transformer losses | ❌ No (predictable, follows formulas)

**Non-Technical Loss (NTL)** | Theft | Meter bypass, tampering, data spoofing | ✅ Yes (creates anomalous patterns)

**India loses ≈₹1.5 lakh crore/year** to NTL. Globally, it's a $96 billion problem.

### Why Can't Humans Find It?

Manual inspection covers only **2-5% of meters/year**

Skilled thieves make it visually undetectable

Inspections are expensive (₹500-2000 per visit)

Some theft techniques (data spoofing) leave **zero physical evidence**

### Our Solution

Use smart meter **telemetry data** (consumption, voltage, current, power factor, frequency, tamper flags) — data that's already being collected — and analyze it with **ML + rule-based heuristics** to find the needles in the haystack.

---

## 2. The Dataset — SGCC

### What is SGCC?

**State Grid Corporation of China (SGCC)** — the gold-standard benchmark dataset for electricity theft research.

Property | Value

Consumers | 42,372

Time Span | 1,035 days (daily kWh readings)

Thieves (labeled FLAG=1) | ~3,615 (8.5%)

Honest (labeled FLAG=0) | ~38,757 (91.5%)

Missing Values | ~2% (realistic)

### Why This Dataset?

1. **It's real** — actual smart meter data from Chinese utility

2. **It's labeled** — human inspectors + investigations confirmed who was stealing

3. **It's the academic benchmark** — every research paper compares against it

4. **It has class imbalance** — realistic 8.5% theft rate, exactly what real-world looks like

### What If We Don't Have It?

Our pipeline also generates **synthetic SGCC-style data** that mimics the same statistical properties — with seasonal patterns (summer AC usage spikes), weekday/weekend variation, and 6 distinct theft attack types.

---

## 3. The 6 Input Dimensions — What the Smart Meter Sends

Every smart meter reading that enters our system contains **6 data dimensions**:

# | Dimension | Unit | Normal Range | What It Measures

1 | **Consumption (kWh)** | kilo-watt hours | 0.5 – 20 kWh/hr (residential) | How much electricity was consumed in this hour

2 | **Voltage (V)** | Volts | 220V ± 30V (India: 190–250V) | The electrical pressure in the line

3 | **Current (A)** | Amperes | 0.5 – 30A (residential) | The flow of electrons through the meter

4 | **Power Factor (PF)** | Ratio (0–1) | 0.85 – 1.0 | How efficiently electricity is being used (Real Power ÷ Apparent Power)

5 | **Frequency (Hz)** | Hertz | 49.5 – 50.5 Hz (India) | The grid's oscillation rate — extremely stable normally

6 | **Tamper Flag** | Boolean (true/false) | false | Hardware sensor in the meter that detects physical opening/shaking

### Why 6 Dimensions? Why Not Just kWh?

Because **different theft techniques disturb different dimensions:**

Meter bypass** → kWh drops, voltage stays normal, current drops to zero → caught by dimensions 1, 3

Voltage manipulation** → voltage outside safe band → caught by dimension 2

Load manipulation** → power factor drops abnormally → caught by dimension 4

Illegal tapping** → frequency deviates → caught by dimension 5

Physical tampering** → tamper flag triggered → caught by dimension 6

**No single dimension catches all theft types.** That's why we analyze all 6 together.

---

## 4. ⭐ The 8 Heuristic Detection Factors — DEEP DIVE

This is the **intellectual core** of the project. Each factor is an independent "detector" that analyzes one aspect of theft. Each produces a score between **0.0 (clean)** and **1.0 (definitely stealing)**.

---

### Factor 1: Consumption Drop 📉 (Weight: 25%)

**What it detects:** Meter bypass during daytime

**The logic:**

"If a residential consumer shows zero consumption during business hours (8 AM–8 PM), that's extremely suspicious. People use AC, lighting, refrigerators — you can't realistically use zero electricity during the day."

**How the code works:**

1. Filter readings to PEAK hours only (8 AM – 8 PM)

2. Count how many peak readings have consumption = 0 kWh

3. Score = min(1.0, (zero_count / total_peak_readings) × 5)

4. ALSO: If avg consumption < 30% of baseline → floor score at 0.7

**Why the ×5 multiplier?** Because even 20% of peak hours being zero is extremely suspicious. The ×5 amplifies this so that 20% zero → score of 1.0.

**Why weight = 25%?** Consumption drop is the **single strongest indicator** of theft. Research shows it appears in 70%+ of all confirmed theft cases.

**Real-world example:** A residential household with baseline 10 kWh/hr suddenly shows 0 kWh from 9 AM to 5 PM for 15 days straight. They bypassed the meter but continued using AC.

---

### Factor 2: Current Anomaly 🔌 (Weight: 15%)

**What it detects:** Physical meter bypass — the meter's current sensor is shunted

**The physics:**

By Ohm's Law: **P = V × I** (Power = Voltage × Current)

If voltage is normal (220V) and current is near-zero (<0.1A), but the consumer is still using electricity (consumption > 0), that's **physically impossible** without theft. The current must be flowing through a bypass wire around the meter's current transformer.

**How the code works:**

1. Count readings where:

current < 0.1A  AND  voltage > 200V  AND  consumption > 0

2. Score = min(1.0, (suspicious_count / total_readings) × 4)

3. ALSO: If current = 0 but consumption > 0.5 kWh → extra boost (×8 multiplier)

**Why ×4 multiplier?** Even 25% of readings showing this pattern is conclusive evidence.

**Why weight = 15%?** Current bypass is a very strong signal, but slightly less common than consumption drops because it requires physical hardware modification.

**Real-world example:** A factory shows 220V voltage, 0.05A current, but 8 kWh consumption. The only explanation: a thick copper wire is bypassing the current transformer inside the meter.

---

### Factor 3: Tamper Detection 🔧 (Weight: 20%)

**What it detects:** Physical tampering with the meter hardware

**The logic:**

Modern smart meters have accelerometers, reed switches, and case-open detectors. When someone physically opens the meter, shakes it, or applies a magnet (to jam the disk), the `tamper_flag` flips to `true`.

**How the code works:**

1. Basic score = min(1.0, tamper_ratio × 10)

(so even 10% tamper rate → score of 1.0)

2. Count maximum CONSECUTIVE tamper flags

If consecutive ≥ 5 → floor score at 0.9

Why consecutive? Random faults create isolated flags (1-2 in a row).

Deliberate tampering creates sustained runs (5+ in a row).

**Why weight = 20%?** Tamper flags are a very reliable physical signal — they come from hardware sensors, not statistical inference. But they can have false positives (storms, accidents).

**Why consecutive matters:**

Pattern | Meaning | Score

`F T F F F T F` | Random noise / power surge | ~0.2

`F T T T T T T` | 6 consecutive — sustained tampering | 0.9

`T T T T T T T` | 100% tampered — definite theft | 1.0

---

### Factor 4: Voltage Anomaly ⚡ (Weight: 10%)

**What it detects:** Voltage manipulation to reduce metered consumption

**The physics:**

Some thieves reduce the voltage entering the meter. Since Energy = V × I × t, reducing voltage while keeping current the same reduces recorded consumption. They use a device called a **"voltage divider"** or tap into a lower-voltage line.

**How the code works:**

1. Safe band: 190V – 250V (India standard)

2. Count readings outside this band

3. Score = min(1.0, (outlier_count / total) × 3)

4. EXTREME voltage check (< 170V or > 270V):

These are 5× weighted because they indicate

deliberate manipulation, not just grid fluctuation

**Why weight = 10%?** Voltage anomalies can have legitimate causes (grid problems, transformer issues during monsoon), so we weight them lower than consumption or tamper signals.

**Key insight:** A legitimate voltage dip affects the **entire neighborhood**. If only ONE meter shows 160V while neighbors show 220V, that's conclusive theft evidence.

---

### Factor 5: Power Factor Anomaly 📊 (Weight: 10%)

**What it detects:** Load manipulation using capacitor banks or inductive loads

**The physics:**

Power Factor = Real Power ÷ Apparent Power

- **PF = 1.0** → Perfect: all energy is doing useful work

- **PF = 0.3** → Very bad: 70% of energy is wasted as reactive power

Thieves sometimes use capacitor banks to deliberately create a low power factor. The meter only records "real power" (watts), but the actual electricity being drawn is much higher (volt-amps). This is a subtle, sophisticated form of theft.

**How the code works:**

1. Count readings where PF < 0.5 (abnormally low)

2. Score = min(1.0, (low_pf_count / total) × 3)

3. Cross-reference check:

Very low PF (< 0.3) + low consumption (< 20% of baseline)

→ This combination is a VERY strong theft signal

→ Uses ×5 multiplier

**Why the cross-reference?** Low PF alone can be legitimate (old motors, industrial equipment). But low PF **combined with** low consumption means someone is drawing power but the meter isn't recording it properly.

---

### Factor 6: Pattern Irregularity 🔀 (Weight: 10%)

**What it detects:** Reversed, chaotic, or artificially manipulated consumption patterns

**The logic:**

Normal electricity usage follows predictable patterns: higher during morning (6-9 AM) and evening (6-10 PM), lower at night. If a consumer shows HIGH consumption at 2 AM but ZERO during daytime, they might be stealing during off-hours when inspectors aren't watching.

**How the code works:**

1. Calculate Coefficient of Variation (CV):

CV = Standard Deviation ÷ Mean

Score = min(1.0, max(0, (CV - 0.5) / 1.5))

High CV (> 0.5) → erratic pattern → suspicious

2. Day/Night Inversion check:

Night average (12 AM – 6 AM) vs Day average (9 AM – 6 PM)

- If night > 2× day → score floor at 0.6

- If day = 0 but night > 0 → score floor at 0.85

3. If mean consumption = 0 → score = 0.8

(Zero mean is inherently suspicious for an active meter)

**Why this catches thieves:**

Pattern | What It Means | Score

Normal curve (peaks at 8 AM, 7 PM) | Legitimate usage | 0.0 – 0.1

Random spikes and drops | Meter data being spoofed | 0.4 – 0.7

High at 2 AM, zero at 2 PM | Stealing when inspectors sleep | 0.6 – 0.85

Zero everything | Either disconnected or fully bypassed | 0.8

---

### Factor 7: Frequency Deviation 🌊 (Weight: 5%)

**What it detects:** Grid-level anomalies caused by illegal high-power tapping

**The physics:**

Grid frequency in India/China is maintained at exactly **50 Hz** (±0.5 Hz). It's one of the most tightly controlled parameters in any electrical grid. Deviations happen when:

- Large illegal loads tap into the grid (factories running off illegal connections)

- Someone is running generators in parallel with the grid without synchronization

**How the code works:**

1. Normal band: 49.0 Hz – 51.0 Hz

2. Count readings outside this band

3. Score = min(1.0, (outlier_count / total) × 4)

**Why weight = only 5%?** Frequency deviations:

Can be caused by legitimate grid issues (generation-demand mismatch)

Affect the entire grid, not just one consumer

Are a weak individual signal but useful when combined with other factors

**When it IS useful:** If a single meter shows 48 Hz while the rest of the grid is at 50 Hz, the meter's internal circuitry may have been tampered with, or there's a local generator interfering.

---

### Factor 8: Flat-line Detection 📏 (Weight: 5%)

**What it detects:** Spoofed or replayed meter data — a hacked meter sending fake readings

**The logic:**

Real electricity consumption is **never** perfectly constant. Even a running refrigerator has compressor cycles. If a meter reports the exact same kWh value for 24 consecutive hours, it's almost certainly sending **pre-recorded fake data**.

**How the code works:**

1. Count unique consumption values in all readings

2. If total readings > 10:

unique_ratio = unique_values / total_readings

Score = min(1.0, max(0, (1 - unique_ratio) × 2 - 0.5))

3. Last-24 check:

If the last 24 readings have ≤ 2 unique values

→ score floor at 0.9

(This catches meters that were recently compromised)

**Why weight = 5%?** Flat-line is a very specific attack type. When present, it's extremely obvious. But it's also rare — most thieves don't have the technical sophistication to hack the meter's firmware.

**Score interpretation:**

Unique Ratio | What It Means | Score

0.95 (95% unique) | Normal — real data | ~0.0

0.50 (50% unique) | Some repetition — possible sensor issue | ~0.25

0.10 (10% unique) | Heavy repetition — likely spoofed | ~0.75

0.02 (same value 24h) | Flat-line — definitely fake data | 0.9+

---

## 5. How the 8 Factors Combine — Weighted Scoring & Boosting

### The Weight Table

Factor | Weight | Why This Weight

Consumption Drop | **25%** | Strongest single indicator — appears in 70%+ of theft cases

Tamper Detection | **20%** | Physical hardware signal — very reliable

Current Anomaly | **15%** | Based on physics (Ohm's Law) — hard to fake

Voltage Anomaly | **10%** | Can have legitimate causes — weighted lower

Power Factor Anomaly | **10%** | Sophisticated theft — useful in combination

Pattern Irregularity | **10%** | Catches behavioral anomalies

Frequency Deviation | **5%** | Weak individual signal — grid-level noise

Flat-line Detection | **5%** | Very specific — rare but conclusive

**Total** | **100%** |

### The Math — Weighted Average

theft_probability = Σ (score_i × weight_i) / Σ weight_i

Example:

Consumption Drop:   0.80 × 0.25 = 0.200

Tamper Detection:   0.90 × 0.20 = 0.180

Current Anomaly:    0.70 × 0.15 = 0.105

Voltage Anomaly:    0.30 × 0.10 = 0.030

Power Factor:       0.20 × 0.10 = 0.020

Pattern Irregular:  0.40 × 0.10 = 0.040

Frequency Deviat:   0.10 × 0.05 = 0.005

Flat-line:          0.00 × 0.05 = 0.000

────────

Sum =       0.580   → 58% theft probability

### Boosting Logic — Why Simple Averaging Isn't Enough

**Rule 1 — Multi-Measure Agreement Boost:**

if 3+ factors score above 0.5:

theft_probability = min(1.0, theft_probability × 1.3)

**Why?** If multiple independent detectors agree something is wrong, the real probability is **higher** than what weighted averaging suggests. This is based on the principle of **independent evidence accumulation**.

In the example above: 3 factors > 0.5 (consumption, tamper, current), so:

0.580 × 1.3 = 0.754  →  75.4% theft probability (critical risk!)

**Rule 2 — Single High-Confidence Floor:**

if any factor ≥ 0.9:

theft_probability = max(0.40, theft_probability)

**Why?** If even ONE factor is screaming "theft" at 0.9+, the overall score should **never** be below 40%. This prevents a scenario where 7 clean factors "wash out" one extremely suspicious one.

### Final Decision

anomaly_flag = True  if  theft_probability ≥ 0.30

Threshold is set at 30% (not 50%) because **false negatives are more costly** than false positives. Missing a thief costs the utility money every day; investigating a false alarm just costs one inspection.

---

## 6. Feature Engineering — The 47+ Engineered Features

Raw smart meter data just says "User used 10 kWh today." That's not enough for ML models. We **engineer** (calculate) new features from the raw data. Our pipeline creates **~47-65 features per consumer** across 6 categories:

### Category 1: Statistical Features (14 features)

Feature | Formula | Why It Matters

`stat_mean` | Average daily consumption | Thieves have lower mean

`stat_std` | Standard deviation | Thieves have erratic patterns

`stat_cv` | std / mean | Coefficient of variation — normalized volatility

`stat_skewness` | Asymmetry of distribution | Theft creates left-skewed data (many zeros)

`stat_kurtosis` | "Peakedness" | Theft creates heavy tails

`stat_median` | Middle value | Robust to outliers

`stat_p10` | 10th percentile | How low the low days are

`stat_p25` | 25th percentile | Lower quartile

`stat_p75` | 75th percentile | Upper quartile

`stat_p90` | 90th percentile | How high the high days are

`stat_iqr` | p75 - p25 | Spread of the middle 50%

`stat_range` | max - min | Total spread

`stat_min` | Minimum value | Thieves often have min = 0

`stat_max` | Maximum value | Normal consumers have seasonal highs

### Category 2: Temporal Features (10 features)

Feature | What It Captures

`temp_weekday_weekend_ratio` | Weekend/Weekday mean ratio — theft patterns often differ

`temp_seasonal_q1` to `q4` | Ratio of quarterly mean to overall mean — theft disrupts seasonality

`temp_avg_mom_change` | Average month-over-month absolute change

`temp_max_mom_change` | Biggest month-over-month swing — theft causes sudden drops

`temp_trend_ratio` | 2nd-half mean / 1st-half mean — theft creates downward trend

`temp_autocorr_lag1` | Day-to-day correlation — theft breaks natural patterns

`temp_autocorr_lag7` | Weekly pattern correlation — honest users are weekly-periodic

### Category 3: Anomaly Indicators (9 features)

Feature | What It Catches

`anom_zero_day_count` | Absolute count of zero-consumption days

`anom_zero_day_ratio` | Fraction of days with zero consumption

`anom_sudden_drops` | Count of days with >60% day-over-day drop

`anom_sudden_drop_ratio` | Fraction of days with sudden drops

`anom_max_flatline_days` | Longest run of identical consecutive values

`anom_flatline_ratio` | Longest flatline / total days

`anom_unique_ratio` | Unique values / total days (low = spoofed)

`anom_volatility_idx` | Rolling CV (std/mean over 30-day windows)

`anom_below_baseline_ratio` | Fraction of days below 30% of baseline

### Category 4: Comparative Features (3 features)

Feature | What It Captures

`comp_zone_deviation` | How many standard deviations from the geographic zone average

`comp_zone_percentile` | Percentile rank within the zone — bottom 5% is suspicious

`comp_ratio_to_zone` | Consumer mean / zone mean — thieves have ratio << 1

### Category 5: Metadata Features (6 features)

Feature | What It Encodes

`meta_is_residential` | 1 if residential, else 0 (one-hot)

`meta_is_commercial` | 1 if commercial, else 0

`meta_is_industrial` | 1 if industrial, else 0

`meta_baseline_kwh` | Historical baseline consumption

`meta_sanctioned_load` | Maximum allowed load from utility

`meta_consumption_to_sanctioned` | Actual / sanctioned ratio

### Category 6: Advanced Derived Features (5 features)

Feature | What It Captures

`adv_entropy` | Information entropy of consumption distribution — low entropy = suspicious

`adv_last_first_ratio` | Last-30-days mean / First-30-days mean — theft causes this to drop

`adv_max_consec_below` | Max consecutive days below 20% of baseline

`adv_weekday_mean` | Average weekday consumption

`adv_weekend_mean` | Average weekend consumption

**Why so many features?** Because ML models work best when given **rich, multi-dimensional views** of the data. A single feature like "zero_day_count" can't distinguish between a vacation and theft. But "zero_day_count" + "zero during peak hours" + "voltage normal" + "current near-zero" + "tamper flag on" → conclusive theft evidence.

---

## 7. Class Imbalance — Why It's Hard & How We Solved It

### The Problem

In SGCC: **91.5% honest, 8.5% thieves.**

If an ML model just always predicts "Honest," it's already **91.5% accurate.** But it catches zero thieves. This is the **Accuracy Paradox.**

### 3 Techniques We Used

#### 1. SMOTE (Synthetic Minority Over-sampling Technique)

**What:** Creates synthetic (fake but realistic) examples of the minority class (thieves).

**How it works — with math:**

x_new = x_i + λ × (x_zi − x_i),    where λ ∈ [0, 1]

Pick a real thief sample `x_i`

Find its nearest neighbor thief `x_zi` (using k=5 neighbors)

Create a new point somewhere on the line between them

This new synthetic thief looks realistic but is a brand-new data point

**Result:** Training set goes from 3,615 thieves → ~38,757 thieves (balanced 50/50).

#### 2. Cost-Sensitive Learning (Class Weights)

class_weight = 'balanced'

# Internally computes: weight_theft = n_total / (2 × n_theft)

# If 5000 total, 400 theft → weight_theft = 5000/800 = 6.25×

**What this does:** The model's loss function penalizes missing a thief **6.25× more** than falsely accusing an honest person. The model literally "cares more" about getting thieves right.

#### 3. Threshold Optimization

Instead of using the default 50% probability cutoff:

# Test every threshold from 0.01 to 0.99

# Find the one that maximizes F1-score

# Result: often around 0.35-0.45

**Why lower threshold?** Missing a thief (false negative) costs the utility ₹10,000s per year. A false alarm just costs one inspection (₹500). So we set the bar lower.

---

## 8. The 5 Machine Learning Models

We train **5 completely different model architectures** — each sees the same data from a different mathematical perspective:

### Model 1: Random Forest 🌲 (300 trees)

Property | Value

**Type** | Ensemble of Decision Trees

**How it works** | Creates 300 independent decision trees. Each tree votes "theft" or "honest." Majority wins.

**Key hyperparameters** | `n_estimators=300`, `max_depth=20`, `max_features=sqrt`

**Strength** | Tells us **feature importance** — which of the 47 features mattered most

**Weakness** | Can overfit to specific patterns in training data

**In simple words:** Imagine 300 different investigators, each looking at a random subset of evidence. If 200+ say "guilty," the person is guilty.

### Model 2: XGBoost 🚀 (300 boosted trees)

Property | Value

**Type** | Gradient Boosted Decision Trees

**How it works** | Builds trees **sequentially** — each tree focuses on fixing mistakes of the previous tree

**Math** | `Obj(θ) = Σᵢ l(yᵢ, ŷᵢ) + Σₖ Ω(fₖ)` — loss + regularization

**Key hyperparameters** | `learning_rate=0.1`, `max_depth=8`, `reg_alpha=0.1 (L1)`, `reg_lambda=1.0 (L2)`

**Strength** | Best-in-class for tabular data, handles imbalance with `scale_pos_weight`

**Weakness** | Slower to train, more hyperparameters to tune

**In simple words:** Tree 1 makes mistakes. Tree 2 specifically studies those mistakes. Tree 3 studies Tree 2's mistakes. By tree 300, the ensemble is extremely accurate.

### Model 3: Deep Neural Network (DNN) 🧠 (128→64→32)

Property | Value

**Type** | Fully Connected Neural Network

**Architecture** | Input → Dense(128) → BatchNorm → Dropout(0.4) → Dense(64) → BatchNorm → Dropout(0.4) → Dense(32) → BatchNorm → Dropout(0.3) → Sigmoid

**Activation** | ReLU (hidden), Sigmoid (output)

**Optimizer** | Adam (lr=0.001)

**Regularization** | Dropout (40%), BatchNormalization, Early Stopping (patience=10)

**Strength** | Finds complex non-linear patterns that tree models miss

**Weakness** | Black box — hard to explain why it made a decision

**In simple words:** A 3-layer artificial brain that learns patterns too complex for decision trees to capture. Dropout randomly "turns off" 40% of neurons during training to prevent memorization.

### Model 4: LSTM 🔄 (2 stacked layers, 128→64 units)

Property | Value

**Type** | Long Short-Term Memory (Recurrent Neural Network)

**Architecture** | Input → LSTM(128, return_sequences=True) → LSTM(64) → Dense(32) → Sigmoid

**Special ability** | **Memory gates** — can remember events from 30+ days ago

**Strength** | Designed for time-series — captures sequential patterns

**Weakness** | Slowest to train; needs careful sequence formatting

**In simple words:** Unlike regular neural networks that look at all features simultaneously, LSTM reads data **like reading a book** — page by page, remembering the plot from earlier pages.

### Model 5: Isolation Forest 🏝️ (200 trees)

Property | Value

**Type** | Unsupervised Anomaly Detector

**How it works** | Randomly partitions data. Anomalies are **isolated** in fewer splits (shorter tree paths).

**Math** | `s(x, n) = 2^(−E[h(x)] / c(n))` — anomaly score based on path length

**Contamination** | 0.1 (assumes ~10% of data is anomalous)

**Key difference** | **Does NOT use labels.** It doesn't know who's a thief.

**Strength** | Catches **novel** theft techniques never seen in training data

**Weakness** | Lower precision — can flag legitimate anomalies (vacations, renovations)

**In simple words:** Instead of learning "what theft looks like," it learns "what normal looks like" and flags anything weird. This catches new types of theft that weren't in the training data.

---

## 9. The Ensemble Strategy — Soft Voting & Stacking

### Why Not Just Use the Best Model?

Because **no single model is perfect.** Each has blind spots:

Model | Blind Spot

Random Forest | Can't capture complex feature interactions

XGBoost | Might overfit to specific theft patterns

DNN | Needs lots of data; black box

LSTM | Overkill for non-temporal patterns

Isolation Forest | Many false positives

### Soft Voting Ensemble

P_ensemble(theft|x) = (1/M) × Σₘ Pₘ(theft|x)

Each model gives a probability. We average them:

RF: 0.72   XGB: 0.68   DNN: 0.81   LSTM: 0.55   IsoForest: 0.60

→ Ensemble = (0.72 + 0.68 + 0.81 + 0.55 + 0.60) / 5 = 0.672

### Stacking (Advanced)

Instead of simple averaging, we train a **Logistic Regression meta-learner** that learns the **optimal weights** for each model's prediction:

P_stacking = σ(w₁×P_RF + w₂×P_XGB + w₃×P_DNN + w₄×P_LSTM + w₅×P_ISO + b)

Where `σ` is the sigmoid function and `w₁...w₅` are learned weights. If XGBoost is consistently more reliable, the meta-learner gives it a higher weight automatically.

---

## 10. Explainability — SHAP & the Heuristic Breakdown

### The Problem

Inspector: *"The AI says 92% theft probability. WHY?"*

You: *"Because the neural network's hidden layer activations..."*

Inspector: *"I'm not acting on that."*

### Solution 1: SHAP (SHapley Additive exPlanations)

Based on **game theory** (Shapley values from Nobel Prize-winning economics):

"How much did each feature **contribute** to this specific prediction?"

For a consumer predicted at 85% theft:

anom_zero_day_ratio:     +0.23  (high zero-days pushed score UP)

stat_cv:                 +0.18  (erratic pattern pushed score UP)

comp_zone_deviation:     +0.15  (way below zone average pushed score UP)

meta_baseline_kwh:       -0.05  (high baseline pushed score DOWN slightly)

temp_autocorr_lag7:      -0.03  (some weekly pattern, slight DOWN)

### Solution 2: The 8-Measure Heuristic Breakdown

Even when the ML ensemble makes the prediction, the **heuristic always runs** to generate the dashboard breakdown:

Consumption Drop:    ████████████████████  80%

Tamper Detection:    ██████████████████████████  90%

Current Anomaly:     ██████████████████  70%

Voltage Anomaly:     ██████  30%

Power Factor:        ████  20%

Pattern Irregular:   ████████  40%

Frequency Deviation: ██  10%

Flat-line:           0%

This gives operators a **physical explanation** — "The meter shows tampering AND current bypass AND consumption dropped."

---

## 11. The Training Pipeline — End to End

┌─────────────────────────────────────────────────────────────┐

│ PHASE 1: Load SGCC Dataset (or generate synthetic data)     │

│   → 42,372 consumers × 1,035 days                          │

│   → 91.5% honest / 8.5% theft                              │

├─────────────────────────────────────────────────────────────┤

│ PHASE 2: Preprocessing                                      │

│   → Forward/backward fill missing values (< 3 day gaps)     │

│   → Mean imputation for longer gaps                         │

│   → Winsorize outliers at 5th/95th percentile               │

│   → Engineer 47-65 features per consumer                    │

├─────────────────────────────────────────────────────────────┤

│ PHASE 3: Handle Class Imbalance                             │

│   → SMOTE (k=5 neighbors)                                   │

│   → Cost-sensitive class weights                             │

├─────────────────────────────────────────────────────────────┤

│ PHASE 4: Train 5 Models                                     │

│   [1] Random Forest → 300 trees, max_depth=20               │

│   [2] XGBoost → 300 trees, L1/L2 regularization             │

│   [3] DNN → 128→64→32, dropout 0.4, early stopping          │

│   [4] Isolation Forest → 200 trees, contamination=0.1        │

│   [5] LSTM → 2 stacked layers (128→64), dropout 0.3          │

├─────────────────────────────────────────────────────────────┤

│ PHASE 5: Ensemble                                           │

│   → Soft Voting (average of all 5 probabilities)             │

│   → Stacking (Logistic Regression meta-learner)              │

│   → Optimal threshold tuning (maximize F1)                   │

├─────────────────────────────────────────────────────────────┤

│ PHASE 6: Evaluation                                         │

│   → Accuracy, Precision, Recall, F1, AUC-ROC, PR-AUC        │

│   → Confusion Matrix (TN, FP, FN, TP)                       │

│   → 5-Fold Stratified Cross-Validation                       │

├─────────────────────────────────────────────────────────────┤

│ PHASE 7: SHAP Explainability                                │

│   → TreeExplainer on Random Forest                           │

│   → Top 15 features by mean absolute SHAP value              │

├─────────────────────────────────────────────────────────────┤

│ SAVE: ensemble_model.pkl, scaler.pkl, feature_names.json     │

└─────────────────────────────────────────────────────────────┘

---

## 12. 6 Theft Scenarios We Simulate

Our synthetic data generator models 6 distinct real-world theft attack types:

# | Attack Type | What Happens | Real-World Method

1 | **Meter Tampering** | Consumption multiplied by 0.1–0.4 after theft_start day | Thief opens meter and adjusts calibration to under-record

2 | **Bypass** | Zero consumption for random 3-15 day blocks | Thick copper wire bypasses current transformer

3 | **Billing Irregularity** | 30-70% of readings randomly replaced with near-zero values | Thief bribes meter reader or hacks billing software

4 | **Unauthorized Tap** | Sudden permanent drop to near-zero (<5% of baseline) | Direct connection to the grid before the meter

5 | **Flat-line Spoof** | Constant value (10-30% of baseline) forever | Hacked meter firmware sends pre-recorded fake data

6 | **Gradual Decay** | Exponential decay: slowly decreasing to 5% of baseline | Slow mechanical degradation of meter (deliberate)

Each scenario:

Starts at a random point (20%–80% through the timeline)

Maintains realistic seasonality before theft begins

Has small noise added to avoid being "too perfect"

---

## 13. Evaluation Metrics — How We Prove It Works

### Why NOT Just Accuracy?

With 91.5% honest consumers, a model that always says "honest" gets **91.5% accuracy** but catches **zero** thieves. Useless.

### The 6 Metrics We Use

Metric | Formula | What It Measures | Our Goal

**Accuracy** | (TP+TN)/(TP+TN+FP+FN) | Overall correctness | High, but don't trust alone

**Precision** | TP/(TP+FP) | Of all "theft" predictions, how many correct? | Reduce false alarms

**Recall** | TP/(TP+FN) | Of all actual thieves, how many did we catch? | Catch every thief

**F1-Score** | 2×(Prec×Rec)/(Prec+Rec) | Harmonic mean of Precision & Recall | Balanced performance

**AUC-ROC** | Area under ROC curve | Discrimination ability across all thresholds | **Primary metric**

**PR-AUC** | Area under Precision-Recall curve | Performance on the minority class | Critical for imbalance

### Confusion Matrix Explained

Predicted

Honest    Theft

Actual  Honest   TN ✅     FP ⚠️ (false alarm — wasted inspection)

Theft    FN ❌     TP ✅ (caught a thief!)

(MISSED!)

FN is the worst** — a thief we missed continues stealing ₹10,000s/year

FP is tolerable** — one unnecessary inspection costs ₹500

That's why we optimize for **Recall** (minimizing FN)

### 5-Fold Stratified Cross-Validation

We don't just test once. We split the data into 5 equal parts:

Fold 1: Train on [2,3,4,5] → Test on [1]

Fold 2: Train on [1,3,4,5] → Test on [2]

Fold 3: Train on [1,2,4,5] → Test on [3]

Fold 4: Train on [1,2,3,5] → Test on [4]

Fold 5: Train on [1,2,3,4] → Test on [5]

**Stratified** = each fold has the same 8.5% theft ratio. We report the **mean ± std** across all 5 folds.

---

## 14. System Architecture — 4 Microservices

┌──────────────────┐    REST/JSON     ┌──────────────────────┐    HTTP POST     ┌───────────────────┐

│  React Frontend   │ ──────────────▶ │  Node.js/Express API  │ ──────────────▶ │  FastAPI ML Svc    │

│  (Vite, Dashboard)│ ◀────────────── │  (Auth, CRUD, Stats)  │ ◀────────────── │  (Heuristic + ML)  │

│  Port: 5173       │    JSON resp    │  Port: 5000           │    JSON resp    │  Port: 8000        │

└──────────────────┘                  └──────────┬───────────┘                 └───────────────────┘

│ Mongoose ODM

┌───────▼────────┐

│   MongoDB 7     │

│   Port: 27017   │

└────────────────┘

**Why microservices (not monolith)?**

Language separation:** Python is best for ML (scikit-learn, numpy); Node.js is best for API routing and async I/O

Independent scaling:** ML service is CPU-heavy → scale it separately

Independent deployment:** Update the ML model without restarting the API

Fault isolation:** ML service crash doesn't bring down the API

---

## 15. The Triple-Layer Fallback — Why the System Never Fails

Layer 1: ML Ensemble (ensemble_model.pkl)

│ If model file exists and loads successfully

│ → Use RF + XGBoost + Isolation Forest + meta-learner

│ → Highest accuracy

│

├── FAIL? ──▶ Layer 2: Python Heuristic (model.py)

│              │ 8-measure deterministic engine

│              │ → No ML model needed, just math rules

│              │ → Still produces anomaly breakdown

│              │

│              ├── FAIL? ──▶ Layer 3: JavaScript Fallback (analyze.js)

│              │              │ Same 8-measure algorithm, reimplemented in JS

│              │              │ → Runs inside Node.js backend

│              │              │ → Zero external dependencies

│              │              │ → System ALWAYS returns a result

**Why maintain code in 2 languages?** Normally this violates DRY principle. But for **critical infrastructure**, availability trumps code elegance. The utility must ALWAYS get an answer.

---

## 16. Data Flow — From Button Click to Result

User clicks "Analyze Anomalies" on a meter

│

▼

Frontend: POST /api/analyze/:meterId

│ JWT token attached via Axios interceptor

▼

Backend Auth Middleware: Verify JWT → extract company ID

│

▼

Backend: Verify meter ownership → Meter.findOne({ _id, company })

│

▼

Backend: Fetch last 30 days of readings

│ Reading.find({ meter, timestamp: { $gte: 30_days_ago } })

│ Returns up to ~720 hourly readings

▼

Backend: Compute statistical features for all 6 dimensions

│ For each: mean, std, min, max

│ Plus: tamper_ratio, baseline_consumption, consumer_type

▼

Backend: POST to ML Service (http://ml-service:8000/predict)

│ Timeout: 15 seconds

│ Payload: { meter_id, features, readings[] }

▼

ML Service: Pydantic validates all 720+ readings

│ (consumption ≥ 0, voltage ≥ 0, PF 0–1, etc.)

▼

ML Service: Try ensemble model → if unavailable → 8-measure heuristic

│

├── Each factor analyzed independently (0–1 score)

├── Weighted combination → boosting applied

├── Risk level classified (low/medium/high/critical)

▼

ML Service returns:

│ { theft_probability, anomaly_flag, anomaly_breakdown{8 scores},

│   source, confidence, risk_level, model_scores[], shap[] }

▼

Backend: If anomaly_flag == true

│ → Create Alert in MongoDB with full breakdown

▼

Frontend: Display result

│ → Anomaly: Red banner + 8 animated progress bars

│ → Clean: Green "All Clear" banner + toast notification

---

## 17. Risk Classification — 4 Tiers

Risk Level | Probability Range | Color | Dashboard Action

🟢 **Low** | 0% – 29% | Green | No action needed

🟡 **Medium** | 30% – 49% | Amber | Schedule inspection within 30 days

🟠 **High** | 50% – 74% | Orange | Schedule inspection within 7 days

🔴 **Critical** | 75% – 100% | Red | Immediate field investigation

**Why 30% threshold (not 50%)?**

Cost-benefit analysis:** Missing a thief costs ₹50,000+/year. A false alarm costs ₹500 (one inspection).

Ratio: 100:1** — it's 100× more expensive to miss a thief than to investigate a false alarm

So we set the bar LOW to catch as many thieves as possible

---

## 18. Summary to Memorize

"We built **PowerGuard**, a full-stack ML-powered electricity theft detection system. It analyzes **6-dimensional smart meter telemetry** (consumption, voltage, current, power factor, frequency, tamper flags) using two parallel detection approaches:

**1) An 8-measure weighted heuristic engine** that produces explainable per-factor scores (consumption drop 25%, tamper detection 20%, current anomaly 15%, etc.) with multi-measure boosting logic.

**2) A 5-model ML ensemble** (Random Forest, XGBoost, DNN, LSTM, Isolation Forest) combined via soft voting and stacking, trained on the **SGCC benchmark dataset** (42,372 consumers) with **SMOTE** for class imbalance handling and **SHAP** for explainability.

We engineered **47+ features** across 6 categories (statistical, temporal, anomaly, comparative, metadata, advanced) and achieved strong detection performance with **5-fold cross-validation**.

The system runs as a **4-service microservice architecture** (React + Node.js + FastAPI + MongoDB) with a **triple-layer fallback** (ML ensemble → Python heuristic → JavaScript heuristic) ensuring zero downtime. The dashboard provides operators with real-time risk classification across 4 tiers and one-click bulk analysis of entire meter networks."

---

[!TIP]

**Interview tip:** Don't memorize this word-for-word. Understand the **WHY** behind every decision. For any component, you should naturally explain:

**What?** → **Why?** → **How does it work technically?** → **What alternatives did you consider?** → **What's the trade-off?**